POTBot

Automated Underwater Camera System

Operations Manual

Firmware Version 10

Open-Source Freeware

github.com/sdelestang/POTBot

2026

# Contents

Contents	2

Introduction	3

System Overview	3

Hardware Components	3

Rear Panel Connections	3

Operational Flow	3

LED Status Indicators	4

Software Installation	5

Arduino IDE	5

Board Support	5

Libraries	5

Programming the POTBot	6

Loading Firmware	6

Setting the Real-Time Clock	6

Configurable Parameters	6

Field Deployment	8

Pre-Deployment Checklist	8

Deployment Procedure	8

Operational Behaviour	8

Data Retrieval	9

Downloading Video	9

Downloading Metadata	9

Serial Monitor Commands	9

Data Storage on Teams	9

Mobius Camera Configuration	10

Setup Procedure	10

Troubleshooting	11

POTBot Does Not Power On	11

No GPS Lock	11

Camera Does Not Record	11

Serial Monitor Shows No Output	11

Firmware Upload Fails	11

Appendix: Pin Assignments	12

# Introduction

The POTBot is an automated underwater camera and environmental monitoring system designed for use in crustacean fisheries research. It is deployed inside lobster pots (fishing traps) to record video footage of pot contents, log water temperature, and capture GPS coordinates upon retrieval.

The system operates autonomously once powered on. A conductivity-based wet switch detects immersion, triggering video recording. Upon retrieval from the water, the unit activates its GPS module to log the pot’s location. Temperature is recorded periodically while the unit remains submerged.

This manual covers software installation, firmware configuration, field deployment, data retrieval, and camera setup. It applies to POTBot firmware version 10 (PB_CodeV10).

# System Overview

## Hardware Components

| Component | Description |
| --- | --- |
| Microcontroller | Adafruit ItsyBitsy M0 Express |
| Camera | Mobius ActionCam |
| GPS Module | Serial GPS (9600 baud, via Serial1) |
| Real-Time Clock | Gravity RTC module (I²C) |
| Temperature Sensor | Dallas DS18B20 (OneWire, pin A1) |
| Flash Storage | Adafruit SPI Flash (onboard, FAT filesystem) |
| Status LED | Adafruit DotStar (single NeoPixel) |
| Wet Switch | Conductivity-based (analog read with digital excitation) |

## Rear Panel Connections

The rear panel of each POTBot unit provides three connections, from left to right:

- Micro-USB for the ItsyBitsy M0 microcontroller (firmware upload and serial data access).

- Battery power connector (centre).

- Micro-USB for the Mobius camera (video file access).

## Operational Flow

The diagram below illustrates the POTBot’s state machine. On power-up, the unit searches for GPS satellites to initialise the module. Once a lock is obtained, it enters a sleep/wake cycle monitoring the wet switch. Detection of water triggers video recording, followed by periodic temperature logging. When retrieved (dry), the GPS activates to record the retrieval position.

Figure 1. POTBot operational state diagram.

## LED Status Indicators

The onboard DotStar LED provides visual feedback on the unit’s current state:

| LED Colour | Meaning |
| --- | --- |
| Green (startup) | Battery voltage above 4.0 V |
| Red (startup) | Battery voltage below 4.0 V — charge or replace battery |
| Yellow | Searching for GPS satellites |
| Purple | GPS lock acquired |
| Off | Normal operation / sleep mode |

# Software Installation

## Arduino IDE

- Download the firmware from GitHub. Navigate to https://github.com/sdelestang/POTBot and click the green “Code” button, then select “Download ZIP”. Save and extract the ZIP file to a convenient location on your computer (e.g. C:\POTBot).

- Download and install Arduino IDE from https://www.arduino.cc/en/software.

- Launch the IDE and follow any prompts to update to the latest version. This can typically be done without administrator permissions.

Note: Board and library installation may fail if the IDE is not updated first. Always update before proceeding.

## Board Support

- Open File → Preferences. In the “Additional Boards Manager URLs” field, enter: https://adafruit.github.io/arduino-board-index/package_adafruit_index.json

- Open Tools → Board → Boards Manager. Search for “Adafruit SAMD” and install the package.

- Restart the IDE. Under Tools → Board → Adafruit SAMD, select “Adafruit ItsyBitsy M0” (may appear as “ItsyBitsy M0 Express” depending on the package version).

## Libraries

The following libraries must be installed via Sketch → Include Library → Manage Libraries:

- OneWire

- DallasTemperature

- Adafruit SleepyDog

- Adafruit SPIFlash (and its dependencies: SdFat – Adafruit Fork, Adafruit NeoPixel)

- TinyGPS++ (or TinyGPS depending on version)

- Adafruit DotStar (and its dependency: Adafruit BusIO)

- RTClib (required for Version 4 and later)

One additional library must be installed manually from a ZIP file: GravityRTC. This file is located in the Github repository: https://github.com/sdelestang/POTBot/tree/master/docs

Download this zip file and install via Sketch → Include Library → Add .ZIP Library and follow the prompts.

To verify installation, check File → Examples — installed libraries should appear with example sketches.

# Programming the POTBot

## Loading Firmware

- Open the firmware sketch. In the extracted POTBot folder, navigate to Version10\PB_CodeV10\PB_CodeV10.ino and double-click to open it in Arduino IDE. Important: Arduino requires that the .ino file resides in a folder with the same name as the sketch. Do not move PB_CodeV10.ino out of its PB_CodeV10 folder, or the IDE will refuse to compile it. If you need to relocate the file, copy the entire PB_CodeV10 folder

- Disconnect the battery from the POTBot.

- Connect the POTBot to the computer using a micro-USB cable plugged into the left-hand port (ItsyBitsy board).

- Under Tools → Port, select the port with the suffix “(Adafruit ItsyBitsy M0)”.

- Verify that Tools → Board is set to “Adafruit ItsyBitsy M0”.

- Click the Upload button (right-arrow icon, top-left). The IDE will compile and then upload the firmware. Progress is shown in the status bar.

- On successful upload, open the Serial Monitor (magnifying glass icon, top-right) to verify the POTBot reports its firmware version and RTC time.

## Setting the Real-Time Clock

The onboard RTC maintains accurate time even when unpowered. It only needs to be set on first use or if the time drifts significantly. The GPS will also automatically correct the RTC when a reliable fix is obtained.

To manually set the RTC:

- In the firmware sketch, locate the RTC calibration line (approximately line 113). Remove the comment markers so it reads:

rtc.adjustRtc(F(__DATE__), F(__TIME__));

- Upload the firmware. This sets the RTC to the computer’s current date and time.

- Immediately re-comment the line so it reads:

//rtc.adjustRtc(F(__DATE__), F(__TIME__));

- Upload the firmware a second time. This prevents the RTC from resetting on every power cycle.

Note: If you forget to re-comment the line, the RTC will reset to the compile time every time the POTBot restarts.

## Configurable Parameters

The following parameters can be adjusted near the top of the firmware sketch (lines 88–91):

| Parameter | Default | Unit | Description |
| --- | --- | --- | --- |
| VidTime | 2400 | seconds | Duration of video recording after deployment |
| GPSTime | 240 | minutes | Duration GPS remains active after retrieval before timeout |
| Dawn | 7 | hour (24h) | Start of daylight window for video assessment |
| Dusk | 17 | hour (24h) | End of daylight window for video assessment |

Ensure each line ends with a semicolon after changing a value.

# Field Deployment

## Pre-Deployment Checklist

- Battery is fully charged (green LED on startup).

- SD card is inserted in the Mobius camera.

- Camera is configured correctly (see Section 8).

- Firmware is loaded and RTC time is correct.

- Previous data has been downloaded and cleared.

## Deployment Procedure

- Connect the battery to the centre power connector on the rear panel.

- The POTBot will power on and display the battery status LED (green = good, red = low).

- The LED will turn yellow as the unit searches for GPS satellites. Wait for the LED to turn purple, indicating a successful GPS lock. After approximately one minute the LED will turn off and the unit is ready.

- Place the POTBot into the lobster pot. When the wet switch detects immersion, the camera will begin recording for the configured video duration.

## Operational Behaviour

Once deployed, the POTBot operates autonomously through the following cycle:

- On immersion: records video for the configured duration (default 40 minutes).

- If deployed during darkness (outside the Dawn–Dusk window): a short additional video (2 minutes) is recorded once daylight is detected, to capture habitat imagery.

- While submerged: records water temperature at scheduled intervals (initially 2 hours after deployment, then at 23:00 daily).

- On retrieval (dry): activates GPS and records the first three position fixes. The GPS remains active for the configured timeout period (default 4 hours).

- If re-deployed before GPS timeout: records the last GPS position and begins a new video cycle.

- If GPS times out without re-deployment: the unit returns to its initial standby state.

Note: The RTC is automatically corrected from the GPS whenever a reliable fix is obtained with four or more satellites.

# Data Retrieval

## Downloading Video

- Connect a micro-USB cable to the right-hand port on the rear panel (Mobius camera).

- The camera will appear as a removable drive on your computer.

- Copy all video files to the appropriate Teams folder.

- Delete the files from the camera to free space for the next deployment.

## Downloading Metadata

- Connect a micro-USB cable to the left-hand port on the rear panel (ItsyBitsy).

- Open Arduino IDE and launch the Serial Monitor (magnifying glass icon) within 7 seconds of connection.

- The POTBot will display its firmware version, RTC date/time, and begin monitoring the wet switch.

- Type Print (case-sensitive) and press Enter. The unit will output all stored deployment data.

- Select and copy all output text. Paste it into a Microsoft Word document, include the skipper name, location, and date in the title, and save to the Teams folder alongside the video files.

- Type Clear to delete all stored data from flash memory, readying the unit for the next deployment.

- Type Run to restart normal operation.

## Serial Monitor Commands

| Command | Action |
| --- | --- |
| Print | Stops the POTBot and prints all stored data (GPS positions, deployment times, temperatures) to the Serial Monitor. |
| Clear | Stops the POTBot and deletes all stored data from the onboard flash memory. |
| Run | Restarts the POTBot and resumes normal wet-switch monitoring. |

## Data Storage on Teams

All data and videos are stored on Microsoft Teams under:

Team Lobster / General / PotBot 2023 / Videos

Create a new folder for each deployment using the naming convention:

[LFB Number]_[DDMMYYYY]_[Camera ID]

For example, POTBot A027 deployed on 12 March 2023 to LFBF020 would be named: F020_12032023_A027

# Mobius Camera Configuration

Each Mobius ActionCam must be configured before its first use with a POTBot. This only needs to be done once per camera unless settings are reset.

## Setup Procedure

- Download and install the mSetup software from: https://www.mytempfiles.info/mobius/

- Ensure an SD card is inserted in the camera before connecting it to the computer. Make sure the SD card is formatted to fat32 regardless of SD card size. Windows will not allow this so for larger ( > 32g) use guiformat.exe (free online) http://ridgecrop.co.uk/index.htm?guiformat.htm

- Connect the camera via USB. In mSetup, click the camera line at the top of the window (showing “E Capacity = …”). Note that often camera will appear in the window as a storage device (will show SD card storage capacity) to remedy press the shutter button on camera (front left) once and wait.

- On the first configuration tab, set the file system to match the SD card format. For larger SD cards, it still must be FAT32.

- On the Video 1 tab, configure the recording settings as required for POTBot operation.

- Click “Set Parameters” to save the configuration to the camera.

Note: When configuring multiple cameras, close and reopen mSetup between each camera to avoid cross-contamination of settings.

# Troubleshooting

## POTBot Does Not Power On

- Verify the battery is connected to the centre port and is charged.

- Check the battery voltage: a red LED on startup indicates low voltage. Replace or charge the battery.

## No GPS Lock

- Ensure the POTBot is outdoors with a clear view of the sky during initial GPS acquisition.

- The yellow LED will remain active while searching. Allow up to several minutes for a cold start.

- If the GPS consistently fails to lock, verify that the GPS module is connected to pin 5 and Serial1 is active.

## Camera Does Not Record

- Confirm the Mobius camera has an SD card inserted and is properly configured (see Section 7).

- Check the camera USB connection to the POTBot mainboard.

- Verify the wet switch is functioning: connect via Serial Monitor and observe whether the POTBot detects immersion.

## Serial Monitor Shows No Output

- The Serial Monitor must be opened within 7 seconds of connecting the USB cable.

- Ensure the correct COM port is selected under Tools → Port.

- Verify the baud rate is set to 9600 in the Serial Monitor.

## Firmware Upload Fails

- Ensure the correct board (Adafruit ItsyBitsy M0) and port are selected.

- Disconnect the battery before uploading.

- Try double-pressing the reset button on the ItsyBitsy to enter bootloader mode.

# Appendix: Pin Assignments

| Variable | Pin | Function |
| --- | --- | --- |
| curr_read | 0 (analog) | Wet switch: analog read for current sensing |
| temp_read | A1 | Dallas DS18B20 temperature sensor (OneWire) |
| curr_out | 11 | Wet switch: digital excitation output |
| curr_quite | 12 | Wet switch: reference current (held LOW) |
| camera | 7 | Mobius camera trigger |
| GPS | 5 | GPS module power control |
| LED | 13 | Onboard LED |
| pwrlsensor | 9 | Light sensor power |
| lsensor | A3 | Light sensor analog input |
| Voltage | A2 | Battery voltage monitor (via voltage divider) |